1. How did implementing debouncing improve the reliability of switch inputs?
Mechanical switches like SW1 and SW2 do not transition cleanly from open to closed. When pressed, the contacts bounce, generating multiple rapid on/off signals. Without debouncing, the program could misread a single press as multiple presses, causing the counter to increment or decrement unexpectedly.
By adding a small delay and verifying the switch state, the debouncing ensures that each press is counted only once, making the counter output stable and predictable.

2. Why is it necessary to configure pull-up resistors for SW1 and SW2?
Pull-up resistors ensure that the input pins (PF4 and PF0) have a defined voltage when the switches are not pressed. Without them, the pins would float, potentially reading as random high or low signals, which could cause the counter to behave erratically.
The internal pull-up resistors connect the pins to logic high (3.3V) when unpressed. Pressing the switch connects the pin to ground, generating a reliable logic low signal.

3. How does using modular functions improve the readability and maintainability of your code?
Functions like Set_Up(), Set_Down(), Display_Number(), PortB_Init(), and PortF_Init() encapsulate specific tasks.

- Improves readability: main() becomes clean and easy to understand because the logic is abstracted.

- Improves maintainability: Changing the behavior of incrementing, decrementing, or display logic only requires updating the corresponding function without affecting the rest of the code.

- Encourages code reuse and prevents repeated code blocks.

4. Suppose you needed to control external LEDs instead of the onboard ones. What changes would be required in the circuit and code?

- Circuit changes: Connect the external LEDs to the appropriate GPIO pins, using current-limiting resistors to prevent damage.

- Code changes: Update the Display_Number() function or the GPIO initialization to point to the pins connected to the external LEDs instead of PB0-PB3. Ensure GPIOx->DIR and GPIOx->DEN are set correctly for the new pins.

- Optionally, modify the mapping if the LEDs represent different bits or patterns.

5. If the counter needed to display numbers beyond 0-9, what modifications would you make?

- Update the counter variable to handle a larger range (e.g., uint8_t or uint16_t).

- Change Display_Number() to drive multiple digits, either by:

  - Using multiple 7-segment displays with digit multiplexing, or

  - Using a BCD to 7-segment decoder like 74LS47 for each digit.

- Modify the increment and decrement logic to wrap appropriately for the new range (e.g., 0–99 for two digits).

- Ensure timing and display update logic can handle multi-digit output.