MENG25301 — Microcontrollers Cheat Sheet  TM4C123GH6PM (ARM Cortex-M4)  |  GPIO · UART · I2C · SPI · Timers · Interrupts · ADC · PWM

▸ GPIO — General Purpose I/O

Clock & Pin Setup

SYSCTL->RCGCGPIO |= (1<<5); // A=0,B=1,C=2,D=3,E=4,F=5

while(!(SYSCTL->PRGPIO & (1<<5)));  // wait ready

GPIOF->DIR  |= 0x0E;  // PF1/2/3 OUTPUT (1=out,0=in)

GPIOF->DIR  &= ~0x11; // PF0/4 INPUT

GPIOF->DEN  |= 0x1F;  // digital enable (required)

GPIOF->PUR  |= 0x11;  // pull-up (buttons active-LOW)

GPIOF->PDR  |= 0x04;  // pull-down

GPIOF->DATA ^= 0x04;  // toggle PF2 (Blue LED)

Alternate Function (e.g. UART PA0/PA1)

GPIOA->AFSEL |= 0x03;

GPIOA->PCTL=(GPIOA->PCTL&~0xFF)|0x11; // UART=1

GPIOA->DEN  |= 0x03;

⚑ PF0 LOCKED: write 0x4C4F434B->LOCK, set CR bit0

▸ UART — Asynchronous Serial

Frame: [START | 5-8 data | PARITY? | 1-2 STOP]

Baud Rate  (SysClk=16MHz, Baud=9600)

ClkDiv = SysClk / (16 x Baud) = 16M/16/9600 = 104.167

IBRD = 104  (integer part)

FBRD = round(0.167 x 64 + 0.5) = 11

| Reg | Purpose | Key bits |
| --- | --- | --- |
| UARTIBRD | Integer baud divisor | — |
| UARTFBRD | Fractional baud div | — |
| UARTLCRH | Line control | WLEN[6:5],FEN[4],STP2[3] |
| UARTCTL | Control | UARTEN[0],HSE[5],RXE[8],TXE[9] |
| UARTDR | Data R/W | bits[7:0] |
| UARTFR | Flags | TXFF[5],RXFE[4],BUSY[3] |

WLEN: 00=5b 01=6b 10=7b 11=8b  |  LCRH=0x60→8N1

Init (UART0, PA0/PA1, 9600 8N1)

SYSCTL->RCGCUART|=1; SYSCTL->RCGCGPIO|=1;

GPIOA->DEN=0x03;GPIOA->AFSEL=0x03;GPIOA->PCTL=0x11;

UART0->CTL =0;       // 1. disable

UART0->IBRD=104;UART0->FBRD=11; // 2. baud

UART0->LCRH=0x60;    // 3. 8N1 no FIFO

UART0->CTL =0x301;   // 4. en+TXE+RXE

▸ SPI (SSI) — 4-Wire Synchronous Full-Duplex

Wires: MOSI MISO SCLK SS(CS)  Master drives clock

| Mode | CPOL |  | CPHA | Sample Edge |
| --- | --- | --- | --- | --- |
| 0 | 0 |  | 0 | Rising / Change:Falling |
| 1 | 0 |  | 1 | Falling / Change:Rising |
| 2 | 1 |  | 0 | Falling / Change:Rising |
| 3 | 1 |  | 1 | Rising / Change:Falling |
| Module |  | Pins |  |  |
| SSI0 |  | PA2-PA5 |  |  |
| SSI1 |  | PF0-PF3 / PD0-PD3 |  |  |
| SSI2 |  | PB4-PB7 |  |  |
| SSI3 |  | PD0-PD3 |  |  |

Config Steps

1. SYSCTL->RCGCSSI|=(1<<n);  // clock SSIn

2. SSI0->CR1 &= ~2;          // disable

3. Set CPSR prescaler & CR0 (mode/size/rate)

4. SSI0->CR1 &= ~4;          // master (bit2=0)

5. SSI0->CR1 |= 2;           // enable

6. Assert SS; wait TNF(SR bit1); write SSI0->DR

7. Wait TFE & !BSY; deassert SS

▸ I2C — 2-Wire Half-Duplex Multi-Master

Lines: SDA + SCL  open-drain, 4.7kΩ pull-ups

Protocol: START→[7b ADDR][R/W]→ACK→[8b DATA]→ACK→STOP

START: SDA low while SCL high

STOP:  SDA high while SCL high

ACK:   receiver pulls SDA low on 9th clock

Up to 128 devices (7-bit addressing)

| Reg |  | Purpose |  |
| --- | --- | --- | --- |
| I2CMSA |  | Master Slave Addr — addr[7:1]+R/W[0] |  |
| I2CMCS |  | Control/Status — RUN,START,STOP,ACK |  |
| I2CMDR |  | Master Data Register (TX/RX byte) |  |
| Module | SCL |  | SDA |
| I2C0 | PB2 |  | PB3 |
| I2C1 | PA6 |  | PA7 |
| I2C2 | PE4 |  | PE5 |
| I2C3 | PD0 |  | PD1 |

⚑ Init: RCGCI2C, GPIO AFSEL/PCTL/DEN/ODR, I2CMCR=0x10

▸ Timers — SysTick (24-bit down-counter)

| Reg | Purpose | Key Bits |
| --- | --- | --- |
| CTRL | Control | [0]=EN [1]=INTEN [2]=CLK_SRC |
| LOAD | Reload | [23:0] max=0xFFFFFF=16,777,215 |
| VAL | Current | Write any → clear to 0 |

Formulas

Delay = (N+1)/sysclk   =>   N = Delay*sysclk - 1

1ms@16MHz: N=15999  |  Max@16MHz: ~1.049s

For >1s: loop multiple 1-second countdowns

Polling Pattern

SysTick->CTRL=0;           // disable

SysTick->CTRL=0x05;        // EN+sysclk, no IRQ

void delay_ms(uint32_t ms){

for(uint32_t i=0;i<ms;i++){

SysTick->LOAD=15999; SysTick->VAL=0;

while(!(SysTick->CTRL&0x10000));}}

Free-Run + Overflow ISR (elapsed time)

SysTick->LOAD=0x00FFFFFF; SysTick->VAL=0;

SysTick->CTRL=0x07; // EN+INTEN+sysclk

void SysTick_Handler(void){overflows++;}

// ticks=dOverflows*(0xFFFFFF+1)+(t1-t2)

// time_us = ticks/16  (16ticks/us@16MHz)

GPTM: 12 blocks, supports PWM/capture/count/compare/IRQ

▸ Interrupts & NVIC

3-Step Enable (example: PORTF)

// 1. Peripheral level

GPIOF->IM |= 0x11;        // unmask PF0 & PF4

// 2. NVIC level

NVIC->ISER[0]|=(1<<30);  // IRQ30=PORTF

// 3. Global

__enable_irq();   // __disable_irq() to mask

GPIO Trigger Registers

GPIOF->IS  &=~0x11; // 0=edge,  1=level

GPIOF->IBE &=~0x11; // 0=single edge

GPIOF->IEV &=~0x11; // 0=falling, 1=rising

| IS | IEV | Trigger |
| --- | --- | --- |
| 0 | 0 | Falling edge |
| 0 | 1 | Rising edge |
| 1 | 0 | Low level |
| 1 | 1 | High level |

ISR Template

void GPIOF_Handler(void){

if(GPIOF->MIS&0x10){/* PF4/SW1 */}

if(GPIOF->MIS&0x01){/* PF0/SW2 */}

GPIOF->ICR|=0x11; // MUST clear flag!

}

Priority:  NVIC->IP[n] = prio<<5  (0=high, 7=low)

| IRQ | Device | IRQ | Device |
| --- | --- | --- | --- |
| 0 | GPIO Port A | 4 | GPIO Port E |
| 1 | GPIO Port B | 5 | UART0 |
| 2 | GPIO Port C | 8 | I2C0 |
| 3 | GPIO Port D | 10 | PWM0 Gen0 |
| 14 | ADC0 Seq0 | 19 | Timer 0A |
| 17 | ADC0 Seq3 | 30 | GPIO Port F |

Auto-pushed on IRQ: R0-R3, R12, LR, PC, xPSR

Flow: IRQ→NVIC→auto-push→vector lookup→ISR→BX LR

▸ ADC — Analog to Digital Converter

Formulas

Levels = 2^n

Resolution = Vref/2^n  (V/LSB)

Code = Vin*(2^n-1)/Vref

Vin  = (Vref*K)/(2^n-1)   [K=ADC value]

Nyquist: Fs >= 2*Fmax

TM4C: 12-bit, Vref=3.3V, step=0.806mV

LM34: Temp(F)=SSFIFO3*330/4096

Resolution (Vref=5V)

8-bit:19.53mV  10-bit:4.88mV  12-bit:1.22mV

| AIN | Pin | AIN | Pin |
| --- | --- | --- | --- |
| AIN0 | PE3 | AIN6 | PD1 |
| AIN1 | PE2 | AIN7 | PD0 |
| AIN2 | PE1 | AIN8 | PE5 |
| AIN3 | PE0 | AIN9 | PE4 |
| AIN4 | PD3 | AIN10 | PB4 |
| AIN5 | PD2 | AIN11 | PB5 |

SS3 Init (single sample, software trigger)

SYSCTL->RCGCADC|=1;  SYSCTL->RCGCGPIO|=(1<<4);

GPIOE->AFSEL|=0x08; GPIOE->DEN&=~0x08;

GPIOE->AMSEL|=0x08;         // analog enable

ADC0->ACTSS &=~8;           // disable SS3

ADC0->EMUX  &=~0xF000;      // SW trigger

ADC0->SSMUX3 =0;            // AIN0

ADC0->SSCTL3 =6;            // IE0+END0

ADC0->ACTSS |=8;            // enable SS3

Read Sample

ADC0->PSSI|=8;              // start conversion

while(!(ADC0->RIS&8));      // wait INT3

result=ADC0->SSFIFO3&0xFFF; // 12-bit result

ADC0->ISC=8;                // clear flag

MENG25301 — Microcontrollers Cheat Sheet  TM4C123GH6PM (ARM Cortex-M4)  |  GPIO · UART · I2C · SPI · Timers · Interrupts · ADC · PWM

▸ PWM — Pulse Width Modulation

Formulas (Count-Down Mode)

DC% = TON/TPWM * 100%

LOAD = PWM_clk / freq

CMPA = (1 - DC%) * LOAD

e.g. 1kHz 25%@16MHz: LOAD=16000,CMPA=12000

e.g. 5kHz 60%@16MHz: LOAD=3200, CMPA=1280

| Reg | Purpose |
| --- | --- |
| PWMxLOAD | Period (sets frequency) |
| PWMxCMPA/B | Compare (duty cycle) |
| PWMxCTL | Start/stop; bit1=up-down mode |
| PWMxGENA/B | Output action on events |
| PWM0/1->ENABLE | Route to output pin |

PWM0 & PWM1  |  4 generators each  |  8 outputs each

Example: M1PWM7 on PF3 — 1kHz 50%DC

SYSCTL->RCGCPWM|=2;          // clock PWM1

SYSCTL->RCGCGPIO|=(1<<5);    // clock Port F

SYSCTL->RCC &=~(1<<20);      // clear USEPWMDIV

GPIOF->AFSEL|=0x08;

GPIOF->PCTL=(GPIOF->PCTL&~0xF000)|0x5000;// M1PWM7=5

GPIOF->DEN|=0x08;

PWM1->_3_CTL =0;             // disable gen3

PWM1->_3_GENB=0x0C8;         // HI@load, LO@CMPB

PWM1->_3_LOAD=16000;         // 16MHz/1kHz

PWM1->_3_CMPB=8000;          // (1-0.5)*16000

PWM1->_3_CTL =1;             // enable gen3

PWM1->ENABLE|=(1<<7);        // enable M1PWM7

⚑ PF3 PCTL: GPIO=0,UART=1,SSI=2,PWM(M1PWM7)=5

▸ Quick Reference

Port F Pin Map

| Pin | Mask | Device | Dir |
| --- | --- | --- | --- |
| PF0 | 0x01 | SW2 (LOCKED) | In pull-up |
| PF1 | 0x02 | Red LED | Output |
| PF2 | 0x04 | Blue LED | Output |
| PF3 | 0x08 | Green LED/PWM | Output |
| PF4 | 0x10 | SW1 | In pull-up |

Minimal LED Blink

SYSCTL->RCGCGPIO|=(1<<5);

while(!(SYSCTL->PRGPIO&(1<<5)));

GPIOF->DIR|=0x04; GPIOF->DEN|=0x04;

while(1){GPIOF->DATA^=0x04;delay_ms(500);}

Read Active-LOW Button

if(!(GPIOF->DATA&0x10)){/* SW1 pressed */}

Parity Bit Calculation

Even: total 1s (data+parity) = EVEN

Odd:  total 1s (data+parity) = ODD

10111010 has 5 ones → even parity=1 (total 6)

10110010 has 4 ones → odd  parity=1 (total 5)

SysTick CTRL Bits

| Bit | Name | Effect |
| --- | --- | --- |
| 0 | ENABLE | Enable SysTick |
| 1 | INTEN | IRQ on underflow |
| 2 | CLK_SRC | 1=sysclk, 0=int osc |
| 16 | COUNT | Set on underflow (r/o) |

0x05=poll mode (EN+CLK)  0x07=IRQ mode (+INTEN)

ADC Code Examples (Vin=3V, Vref=5V)

8-bit:  3*255/5  = 153   (0x99)

10-bit: 3*1023/5 = 614   (0x266)

12-bit: 3*4095/5 = 2457  (0x999)

UART Flag Register (UARTFR)

| Bit | Name | Meaning |
| --- | --- | --- |
| 3 | BUSY | UART transmitting |
| 4 | RXFE | RX FIFO empty |
| 5 | TXFF | TX FIFO full |
| 6 | RXFF | RX FIFO full |
| 7 | TXFE | TX FIFO empty |

Serial Interface Comparison

| Proto | Wires | Duplex | Sync | Speed |
| --- | --- | --- | --- | --- |
| UART | 2(TX,RX) | Full | Async | ~115kbps |
| SPI | 4 | Full | Sync | ~10Mbps |
| I2C | 2(SDA,SCL) | Half | Sync | ~400kbps |

▸ Interrupt Vector Table (ARM Cortex-M)

| INT# | Interrupt | Addr |
| --- | --- | --- |
| 1 | Reset (startup) | 0x00000004 |
| 2 | NMI | 0x00000008 |
| 3 | Hard Fault | 0x0000000C |
| 11 | SVCall | 0x0000002C |
| 14 | PendSV | 0x00000038 |
| 15 | SysTick | 0x0000003C |
| 16+ | IRQ0+ (peripherals) | 0x00000040+ |

First 1024 bytes reserved for NVIC (256 x 4 bytes)

Priority: Reset=-3, NMI=-2, HardFault=-1, rest=0-7

ARM Cortex-M Processor Modes

| Mode | Triggered by | Privilege |
| --- | --- | --- |
| Thread | Reset / return from ISR | Priv or Unpriv |
| Handler | Any exception / IRQ | Always Privileged |

Stack pointers: MSP (main) used in Handler mode; PSP (process) in Thread mode

▸ Nested Interrupt Handling

Higher-priority IRQ can interrupt a running ISR

NVIC manages preemption automatically

Interrupt latency = time from IRQ to ISR start

Context save/restore is automatic (hardware)

▸ GPIO Interrupt Registers Summary

| Reg | Purpose |
| --- | --- |
| GPIOIM | Interrupt Mask — enable/disable per pin |
| GPIOIS | Sense — 0=edge, 1=level |
| GPIOIBE | Both Edges — 1=both edges trigger |
| GPIOIEV | Event — 0=falling/low, 1=rising/high |
| GPIORIS | Raw Interrupt Status |
| GPIOMIS | Masked Interrupt Status (use in ISR) |
| GPIOICR | Interrupt Clear — write 1 to clear |

▸ Key Constants & Values

16 MHz sysclk → 1 tick = 62.5 ns

16 MHz sysclk → 16 ticks per microsecond

SysTick max LOAD = 0xFFFFFF = 16,777,215

SysTick max delay@16MHz ≈ 1.049 seconds

TM4C ADC: 12-bit, 0-4095, Vref=3.3V

ADC step size = 3.3V/4096 = 0.806 mV/LSB

UART 9600@16MHz: IBRD=104, FBRD=11

PWM PCTL M1PWM7 on PF3 = value 5

IRQ30 = GPIO Port F

NVIC ISER[0] bit30 → enables IRQ30 (Port F)

Common #include / Peripheral Base Addresses

#include "TM4C123GH6PM.h"

GPIOF base: 0x40025000

UART0 base: 0x4000C000

ADC0  base: 0x40038000

PWM1  base: 0x40029000

SysTick: 0xE000E010 (CorePeripherals)

Configure Interrupt

void GPIOF_Init_Interrupt(void){
  SYSCTL->RCGCGPIO |= (1<<5);
  while(!(SYSCTL->PRGPIO & (1<<5)));

  GPIOF->DIR &= ~0x10;     // PF4 input
  GPIOF->DEN |= 0x10;
  GPIOF->PUR |= 0x10;

  GPIOF->IS  &= ~0x10;     // edge
  GPIOF->IBE &= ~0x10;     // single edge
  GPIOF->IEV &= ~0x10;     // falling edge
  GPIOF->ICR |= 0x10;      // clear flag
  GPIOF->IM  |= 0x10;      // unmask

  NVIC->ISER[0] |= (1<<30); // enable IRQ30
  __enable_irq();
}

Timer (SysTick)

void SysTick_Init(void){
  SysTick->CTRL = 0;
  SysTick->LOAD = 0x00FFFFFF;
  SysTick->VAL  = 0;
  SysTick->CTRL = 0x05; // enable, no interrupt
}

void delay_us(uint32_t us){
  for(uint32_t i=0;i<us;i++){
    SysTick->LOAD = 16-1; // 1us @16MHz
    SysTick->VAL = 0;
    while(!(SysTick->CTRL & 0x10000));
  }
}

⚑ This gives microsecond + millisecond flexibility

Send String

void UART0_SendString(char *str){
  while(*str){
    while(UART0->FR & 0x20); // wait if TX full
    UART0->DR = *str;
    str++;
  }
}

SPI Transfer Function

uint8_t SPI_Transfer(uint8_t data){
  while(!(SSI0->SR & 0x02)); // TNF
  SSI0->DR = data;
  while(SSI0->SR & 0x10);    // wait BSY
  return SSI0->DR;
}

⚑ write then read from SPI device; full duplex explanation

I2C Write + Read Pattern

void I2C_Write(uint8_t addr, uint8_t data){
  I2C0->MSA = (addr<<1);     // write
  I2C0->MDR = data;
  I2C0->MCS = 0x07;          // START + RUN + STOP
  while(I2C0->MCS & 0x01);   // wait busy
}

uint8_t I2C_Read(uint8_t addr){
  I2C0->MSA = (addr<<1) | 1; // read
  I2C0->MCS = 0x07;
  while(I2C0->MCS & 0x01);
  return I2C0->MDR;
}

⚑ Write to sensor; Read from register

ADC → Voltage Conversion

float ADC_to_Voltage(uint16_t adc){
  return (adc * 3.3) / 4095.0;
}

⚑ Converts theory → practical answer

ADC → Temperature

float TempF(uint16_t adc){
  return (adc * 330.0) / 4096.0;
}

PWM Duty Cycle Function

void PWM_SetDuty(uint16_t load, float duty){
  PWM1->_3_CMPB = (1.0 - duty) * load;
}

⚑ PWM_SetDuty(16000, 0.25); // 25%

Main Loop Pattern

int main(void){
  GPIO_Init();
  UART0_Init();
  SysTick_Init();

  while(1){
    if(!(GPIOF->DATA & 0x10)){
      UART0_SendString("Pressed\n");
      delay_us(500000);
    }
  }
}

⚑ write a full program; integration questions

Button Interrupt Trigger

GPIOF->IS  &= ~0x10;
GPIOF->IEV &= ~0x10;   // falling
GPIOF->IM  |= 0x10;
NVIC->ISER[0] |= (1<<30);

UART Receive Char

while(UART0->FR & 0x10);
char c = UART0->DR;

ADC Read (SS3)

ADC0->PSSI |= 8;
while(!(ADC0->RIS & 8));
val = ADC0->SSFIFO3 & 0xFFF;
ADC0->ISC = 8;

INITIALIZATION ORDER

1. Enable clock (SYSCTL->RCGCxxx)
2. Wait ready (PRxxx)
3. Configure GPIO (DIR, DEN, AFSEL, PCTL)
4. Configure peripheral registers
5. Enable peripheral
6. Enable NVIC (if interrupt)

Bitwise Operations

SET BIT:    reg |=  (1<<n);
CLEAR BIT:  reg &= ~(1<<n);
TOGGLE:     reg ^=  (1<<n);
CHECK:      if(reg & (1<<n))

reg &= ~0x0F;   // clear lower 4 bits
reg |=  0x05;   // set bits 0 and 2

Polling vs Interrupt (MCQ gold)

POLLING:
- CPU waits (while loop)
- simple but wastes CPU

INTERRUPT:
- event-driven
- CPU free until trigger
- requires NVIC + ISR

Edge vs Level Trigger

Edge → one-time event (button press)
Level → continuous while condition true

Falling edge → button press (active LOW)

UART / SPI / I2C — WHEN to use each

UART → simple, 2 devices, long distance
SPI  → fast, short distance, many slaves
I2C  → multiple devices, fewer wires

Throughput / Timing Insight

UART baud = bits/sec
SPI speed = clock rate
I2C speed limited by pull-ups

Higher clock → faster but less reliable

ADC Sampling Concept

Higher sampling rate → better signal accuracy
Aliasing if Fs < 2*Fmax

Interrupt Flow

Event → Flag set → NVIC → ISR → clear flag

- ALWAYS enable clock before config
- ALWAYS clear interrupt flag in ISR
- ALWAYS enable DEN for GPIO
- ALWAYS wait for peripheral ready

MENG25301 — Microcontrollers Cheat Sheet 
TM4C123GH6PM (ARM Cortex-M4) 
GPIO · UART · I2C · SPI · Timers · Interrupts · ADC · PWM
────────────────────────────────────
▸ INITIALIZATION ORDER (ALWAYS)
1. Enable clock (SYSCTL->RCGCxxx)
2. Wait ready (PRxxx)
3. Configure GPIO (DIR, DEN, AFSEL, PCTL)
4. Configure peripheral registers
5. Enable peripheral
6. Enable NVIC (if interrupt)
────────────────────────────────────
▸ BITWISE OPERATIONS
SET:    reg |=  (1<<n);
CLEAR:  reg &= ~(1<<n);
TOGGLE: reg ^=  (1<<n);
CHECK:  if(reg & (1<<n))

MASKING:
reg &= ~0x0F;   // clear bits
reg |=  0x05;   // set bits
────────────────────────────────────
▸ GPIO
SYSCTL->RCGCGPIO |= (1<<5);
while(!(SYSCTL->PRGPIO & (1<<5)));

GPIOF->DIR |= 0x0E;
GPIOF->DIR &= ~0x11;
GPIOF->DEN |= 0x1F;
GPIOF->PUR |= 0x11;

GPIOF->DATA ^= 0x04;

ALT FUNC (UART):
GPIOA->AFSEL |= 0x03;
GPIOA->PCTL = (GPIOA->PCTL & ~0xFF) | 0x11;
GPIOA->DEN  |= 0x03;

⚑ PF0 LOCK: write 0x4C4F434B → LOCK, set CR bit0
────────────────────────────────────
▸ UART
IBRD=104, FBRD=11 (9600 @16MHz)

INIT:
SYSCTL->RCGCUART |=1;
SYSCTL->RCGCGPIO |=1;
GPIOA->DEN=0x03; GPIOA->AFSEL=0x03; GPIOA->PCTL=0x11;

UART0->CTL=0;
UART0->IBRD=104; UART0->FBRD=11;
UART0->LCRH=0x60;
UART0->CTL=0x301;

SEND STRING:
while(*s){
 while(UART0->FR & 0x20);
 UART0->DR = *s++;
}

RECEIVE:
while(UART0->FR & 0x10);
char c = UART0->DR;
────────────────────────────────────
▸ SPI (SSI)
while(!(SSI0->SR & 0x02));
SSI0->DR = data;
while(SSI0->SR & 0x10);
return SSI0->DR;
────────────────────────────────────
▸ I2C
WRITE:
I2C0->MSA = (addr<<1);
I2C0->MDR = data;
I2C0->MCS = 0x07;
while(I2C0->MCS & 1);

READ:
I2C0->MSA = (addr<<1)|1;
I2C0->MCS = 0x07;
while(I2C0->MCS & 1);
data = I2C0->MDR;
────────────────────────────────────
▸ SYSTICK TIMER
Delay = (N+1)/sysclk

INIT:
SysTick->CTRL=0;
SysTick->LOAD=0x00FFFFFF;
SysTick->VAL=0;
SysTick->CTRL=0x05;

DELAY (us):
SysTick->LOAD = 16-1;
SysTick->VAL=0;

while(!(SysTick->CTRL & 0x10000));
──────────────────────────────────

▸ INTERRUPTS
ENABLE:
GPIOF->IM |= 0x10;
NVIC->ISER[0] |= (1<<30);
__enable_irq();

TRIGGER:
GPIOF->IS  &= ~0x10;
GPIOF->IEV &= ~0x10;

ISR:
void GPIOF_Handler(void){
  if(GPIOF->MIS & 0x10){}
  GPIOF->ICR |= 0x10;
}

FLOW:
Event → Flag → NVIC → ISR → clear flag
──────────────────────────────────
▸ ADC
FORMULAS:
Code = Vin*(2^n-1)/Vref
Vin  = (Vref*K)/(2^n-1)

INIT:
SYSCTL->RCGCADC|=1;
SYSCTL->RCGCGPIO|=(1<<4);

GPIOE->AFSEL|=0x08;
GPIOE->DEN&=~0x08;
GPIOE->AMSEL|=0x08;

ADC0->ACTSS &=~8;
ADC0->EMUX  &=~0xF000;
ADC0->SSMUX3 =0;
ADC0->SSCTL3 =6;
ADC0->ACTSS |=8;

READ:
ADC0->PSSI|=8;
while(!(ADC0->RIS&8));
val = ADC0->SSFIFO3 & 0xFFF;
ADC0->ISC=8;

VOLTAGE:
V = (adc * 3.3) / 4095.0;

TEMP:
TempF = (adc * 330.0) / 4096.0;
──────────────────────────────────
▸ PWM
LOAD = PWM_clk / freq
CMP  = (1 - duty) * LOAD

SET DUTY:
PWM1->_3_CMPB = (1.0 - duty) * load;
───────────────────────────────────
▸ QUICK CODE PATTERNS
LED BLINK:
GPIOF->DIR |= 0x04;
GPIOF->DEN |= 0x04;
while(1){GPIOF->DATA ^= 0x04;}

BUTTON:
if(!(GPIOF->DATA & 0x10)){}
──────────────────────────────────
▸ CONCEPTS (MCQ GOLD)

POLLING:
- CPU waits

INTERRUPT:
- event-driven

EDGE vs LEVEL:
- Edge = one event
- Level = continuous
- Falling edge = button press

COMMUNICATION:
UART → simple, long distance  
SPI  → fast  
I2C  → multiple devices  

ADC:
- Fs ≥ 2Fmax (Nyquist)
- aliasing if violated

TIMING:
UART baud = bits/sec
SPI speed = clock
I2C limited by pull-ups
───────────────────────────────────
▸ MUST REMEMBER
- Enable clock FIRST
- Wait for ready
- Enable DEN for GPIO
- Clear interrupt flags

Q: Why use interrupt instead of polling?
A: Saves CPU, event-driven

Q: Why pull-up resistor?
A: Prevent floating input

Q: Why Nyquist?
A: Avoid aliasing

Q: Why SPI faster than I2C?
A: No addressing overhead + push-pull

▸ GPIO BITMASK PATTERNS

ENABLE PORT CLOCK:
A=0x01  B=0x02  C=0x04  D=0x08  E=0x10  F=0x20
Example:
SYSCTL->RCGCGPIO |= 0x03; // A+B
SYSCTL->RCGCGPIO |= 0x31; // A+E+F

────────────────────
SET PINS AS OUTPUT:
GPIOx->DIR |= mask;
GPIOx->DEN |= mask;

Examples:
PA0        → 0x01
PA0,3,6    → 0x49
PB2,3,4    → 0x1C

────────────────────
SET INPUT + PULL-UP:
GPIOx->DIR &= ~mask;
GPIOx->DEN |= mask;
GPIOx->PUR |= mask;

Example:
PD2,3,4 → mask=0x1C

────────────────────
WRITE TO PINS:

TURN ON:
GPIOx->DATA |= mask;

TURN OFF:
GPIOx->DATA &= ~mask;

TOGGLE:
GPIOx->DATA ^= mask;

────────────────────
IMPORTANT:
"WITHOUT AFFECTING OTHER PINS"
→ ALWAYS use |= or &= ~
→ NEVER use =

▸ QUICK MASK BUILDING

Bit n = 1<<n

Examples:
PA3 → (1<<3) = 0x08
PB2,3,4 → (1<<2)|(1<<3)|(1<<4) = 0x1C

MENG25301 — Microcontrollers Cheat Sheet
TM4C123GH6PM (ARM Cortex-M4)
GPIO · UART · I2C · SPI · Timers · Interrupts · ADC · PWM
────────────────────────────────────
▸ INITIALIZATION ORDER (ALWAYS)

1. Enable clock (SYSCTL->RCGCxxx)
2. Wait ready (PRxxx)
3. Configure GPIO (DIR, DEN, AFSEL, PCTL)
4. Configure peripheral
5. Enable peripheral
6. Enable NVIC (if interrupt)
────────────────────────────────────
▸ BITWISE OPERATIONS
SET:    reg |=  (1<<n);
CLEAR:  reg &= ~(1<<n);
TOGGLE: reg ^=  (1<<n);
CHECK:  if(reg & (1<<n))

MASKING:
reg &= ~0x0F;
reg |=  0x05;
────────────────────────────────────
▸ GPIO (CORE PATTERN)
SYSCTL->RCGCGPIO |= (1<<5);
while(!(SYSCTL->PRGPIO & (1<<5)));

GPIOF->DIR |= 0x0E;
GPIOF->DIR &= ~0x11;
GPIOF->DEN |= 0x1F;
GPIOF->PUR |= 0x11;

GPIOF->DATA ^= 0x04;

ALT FUNC (UART):
GPIOA->AFSEL |= 0x03;
GPIOA->PCTL = (GPIOA->PCTL & ~0xFF) | 0x11;
GPIOA->DEN  |= 0x03;

⚑ PF0 LOCK → 0x4C4F434B
────────────────────────────────────
▸ UART (9600 @16MHz)
IBRD=104, FBRD=11

INIT:
SYSCTL->RCGCUART |=1;
SYSCTL->RCGCGPIO |=1;
GPIOA->DEN=0x03; GPIOA->AFSEL=0x03; GPIOA->PCTL=0x11;

UART0->CTL=0;
UART0->IBRD=104; UART0->FBRD=11;
UART0->LCRH=0x60;
UART0->CTL=0x301;

SEND:
while(*s){
while(UART0->FR & 0x20);
UART0->DR = *s++;
}

RECEIVE:
while(UART0->FR & 0x10);
char c = UART0->DR;
────────────────────────────────────
▸ SPI
while(!(SSI0->SR & 0x02));
SSI0->DR = data;
while(SSI0->SR & 0x10);
return SSI0->DR;
────────────────────────────────────
▸ I2C
WRITE:
I2C0->MSA = (addr<<1);
I2C0->MDR = data;
I2C0->MCS = 0x07;
while(I2C0->MCS & 1);

READ:
I2C0->MSA = (addr<<1)|1;
I2C0->MCS = 0x07;
while(I2C0->MCS & 1);
data = I2C0->MDR;
────────────────────────────────────
▸ SYSTICK
INIT:
SysTick->CTRL=0;
SysTick->LOAD=0x00FFFFFF;
SysTick->VAL=0;
SysTick->CTRL=0x05;

DELAY (1us):
SysTick->LOAD = 16-1;
SysTick->VAL=0;
while(!(SysTick->CTRL & 0x10000));
───────────────────────────────────
▸ INTERRUPTS
ENABLE:
GPIOF->IM |= 0x10;
NVIC->ISER[0] |= (1<<30);
__enable_irq();

ISR:
void GPIOF_Handler(void){
if(GPIOF->MIS & 0x10){}
GPIOF->ICR |= 0x10;
}

FLOW:
Event → NVIC → ISR → CLEAR FLAG
───────────────────────────────────
▸ ADC
Code = Vin*(2^n-1)/Vref
Vin  = (Vref*K)/(2^n-1)

READ:
ADC0->PSSI|=8;
while(!(ADC0->RIS&8));
val = ADC0->SSFIFO3 & 0xFFF;
ADC0->ISC=8;

Voltage:
V = (adc * 3.3) / 4095.0;

Temp:
TempF = (adc * 330.0) / 4096.0;
───────────────────────────────────
▸ PWM
LOAD = PWM_clk / freq
CMP  = (1 - duty) * LOAD

PWM1->_3_CMPB = (1.0 - duty) * load;
───────────────────────────────────
▸ QUICK PATTERNS
LED:
GPIOF->DIR |= 0x04;
GPIOF->DEN |= 0x04;
while(1){GPIOF->DATA ^= 0x04;}

BUTTON:
if(!(GPIOF->DATA & 0x10)){}
───────────────────────────────────
▸ MCQ GOLD
Polling = CPU waits
Interrupt = event-driven

Edge = one event
Level = continuous

UART → simple
SPI → fastest
I2C → multi-device

Nyquist: Fs ≥ 2Fmax
───────────────────────────────────
▸ MUST REMEMBER

* Enable clock FIRST
* Wait ready
* Enable DEN
* Clear interrupt flag
───────────────────────────────────
▸ WORKSHEET PATTERNS (EXAM GOLD)

PORT CLOCK MASKS:
A=0x01 B=0x02 C=0x04 D=0x08 E=0x10 F=0x20

Examples:
A+B → 0x03
B+D → 0x0A
A+E+F → 0x31
────────────────────
OUTPUT PINS:
GPIOx->DIR |= mask;
GPIOx->DEN |= mask;

Examples:
PA0 → 0x01
PA0,3,6 → 0x49
PB2,3,4 → 0x1C
────────────────────
INPUT + PULL-UP:
GPIOx->DIR &= ~mask;
GPIOx->DEN |= mask;
GPIOx->PUR |= mask;

Example:
PD2,3,4 → 0x1C
────────────────────
WRITE TO PINS:
ON:     GPIOx->DATA |= mask;
OFF:    GPIOx->DATA &= ~mask;
TOGGLE: GPIOx->DATA ^= mask;

⚑ NEVER use '=' unless resetting ALL pins
────────────────────
MASK BUILDING:

Bit n = (1<<n)

Examples:
PA3 → 0x08
PB2,3,4 → 0x1C

────────────────────
LED CONTROL EXAMPLE (PORT E):

GPIOE->DIR |= 0xFF;
GPIOE->DEN |= 0xFF;

ON PE0,2,5:
GPIOE->DATA |= 0x25;

SWAP PE2→OFF, PE3→ON:
GPIOE->DATA ^= 0x0C;

TURN ON PE1:
GPIOE->DATA |= 0x02;

────────────────────────────────────
────────────────────────────────────

PAGE 2 — DETAILED REFERENCE

(Keep for deeper questions, not memorization)

* Full UART register breakdown
* Full SPI modes table
* I2C protocol sequence
* SysTick math + overflow timing
* NVIC priority + IRQ table
* ADC resolution table + channels
* PWM generator setup example
* Interrupt vector table
* Cortex-M modes + stack behavior

(Use only if exam asks THEORY or LONG FORM)