MENG 4010 - INDUSTRIAL NETWORKING

COMMUNICATION BETWEEN AB PLCS USING PRODUCED/CONSUMED TAGS

Student Name & Number:

Lab Date:

# Objective

In this lab, two Allen-Bradley ControlLogix 1756-L81E PLCs exchange information using produced and consumed tags.

PLC 1 reads the voltage from analog input channel 0 of its analog input module. This voltage input, ranging from 0 to 10 V, is provided by a voltage generator. The raw input value is then transmitted to PLC 2 via a produced tag.

PLC 2 receives the value through a consumed tag and converts it back into a floating-point number representing the actual voltage.

- Upon successful completion of this lab, students will be able to:

- Configure produced and consumed tags between two ControlLogix PLCs

- Read analog input data from a module and share it over Ethernet/IP

- Convert raw analog values to engineering units (e.g., voltage)

- Verify inter-PLC communication using tag monitoring in Studio 5000

# Background

Produced and consumed tags are a method of data exchange between Allen-Bradley ControlLogix PLCs over an Ethernet/IP network. Instead of using traditional messaging instructions (MSG), this method enables real-time, high-speed communication by linking tags directly between controllers. A produced tag is created in one PLC and made available for other controllers to consume. The consuming PLC establishes a connection to the producer and receives updates cyclically without requiring explicit messaging logic.

# Demonstration Requirements

To successfully complete the lab, students must demonstrate the following:

- Tag Configuration

  - Correctly create and configure a produced tag in PLC 1.

  - Correctly configure a corresponding consumed tag in PLC 2, matching the data type, name, and connection parameters.

- Analog Input Reading

  - Read a varying voltage signal (0–10 V) from analog input channel 0 on PLC 1.

  - Show the raw analog value in PLC 1’s tag monitor.

- Data Transfer

  - Verify that the raw analog value from PLC 1 is being successfully transferred to PLC 2 via the consumed tag.

  - Demonstrate real-time updates when the voltage input is changed.

- Data Conversion

  - In PLC 2, convert the raw analog value into a floating-point voltage using the correct scaling formula.

  - Display the scaled voltage in a user-defined tag.

- Monitoring and Verification

  - Use Studio 5000’s online monitoring tools to show:

    - Live updates of the analog input on PLC 1

    - Real-time consumption of the tag on PLC 2

    - Correct scaling of the voltage in PLC 2

- Clean and Organized Program

  - Ensure the ladder logic is neatly organized, well-documented, and correctly downloaded to the PLCs.

# Optional Extension Task (Bonus)

To further enhance your understanding of distributed control systems and data handling across PLCs, complete the following optional tasks:

1. Simulated Alarm Triggering Based on Remote Input

Use the consumed voltage value in PLC 2 to create a simulated alarm system. For example: If the voltage exceeds 8.0 V, trigger a local alarm (turn on a red indicator and activate an alarm bit). Use a timer to latch the alarm for 5 seconds even if the condition clears.

2. Bidirectional Communication

Implement a second data channel going in the opposite direction: Configure a new produced tag in PLC 2 and a corresponding consumed tag in PLC 1. Use PLC 2 to send a status signal such as “Overvoltage” back to PLC 1 based on threshold logic.

# Lab Report Requirements

The lab report must contain:

- Instructor’s Confirmation

- The course instructor’s signature is required to verify the completion of the lab. Please ensure this is included with your report’s submission.

- Assignment Summary

- Provide a clear summary of the assignment, detailing the objectives, a description of the work completed, and your conclusions.

- PLC Program Documentation

- Attach a PDF of the PLC program and the PLC program itself that you created, ensuring it includes your initials whenever applicable.

- The lab report will be assessed based on the lab marking rubric provided on Blackboard. Please review the rubric to ensure all criteria are met.

# Application Photos