MENG 4010 - INDUSTRIAL NETWORKING

SETUP OF IFM AL1320 IO-LINK WITH MQTT AND NODE-RED

Student Name & Number:

Lab Date:

# Objective

In this lab, students will learn how to integrate the IFM AL1300 IO-Link master with a cloud-based MQTT broker and visualize sensor data using the Node-RED dashboard.

Students will:

- Configure the IFM to read IO-Link sensor data.

- Forward real-time data to an MQTT broker.

- Set up Node-RED to acquire sensor data via MQTT.

- Visualize the data on a dashboard.

# Background

MQTT (Message Queuing Telemetry Transport) is a lightweight, publish-subscribe network protocol commonly used in industrial IoT for reliable data communication between field devices and cloud platforms.

Node-RED, a visual programming tool, simplifies the development of automation workflows by enabling users to wire together devices, APIs, and services using a drag-and-drop interface. It also supports native MQTT communication and dashboard creation.

This lab demonstrates an edge-to-cloud integration: sensor data collected by the IO-Link master is published to a cloud MQTT broker and visualized using Node-RED's dashboard.

# Demonstration Requirements

To successfully complete the lab, students must demonstrate the following:

IO-Link Configuration

- Access the IO Link Master IFM AL1300

- Ensure data from the IO-Link sensors is accessible via the IO-Link master’s web interface.

- Verify sensor communication and data via the web dashboard.

MQTT Broker Configuration

- Use a public broker (test.mosquitto.org).

- Subscribe to the appropriate topic. Make sure the topic that you subscribe to has a unique name.

- Verify data is transmitted in real time.

Node-RED Setup & Data Visualization

- Install Node-RED on a local machine.

- Reads sensor values from the AL1300 via MQTT.

- Create a Node-RED dashboard to display live sensor data using a gauge, chart, or numeric widget.

# Optional Extension Task (Bonus)

1. Dashboard-Based Alarm Notification

Enhance the Node-RED dashboard by adding a virtual alarm system:

- Configure a threshold on the incoming sensor value (temperature or vibration data exceeds a predefined threshold).

- Trigger a visual indicator on the dashboard.

2. Cloud Integration

Connect your MQTT flow to a cloud-based IoT platform (ThingsBoard, Adafruit IO, ThingSpeak, HiveMQ Cloud, etc.):

- Forward sensor data to the cloud platform.

- Visualize it using the platform's tools.

- Compare latency and reliability between local MQTT and cloud MQTT communication.

# Lab Report Requirements

The lab report must contain:

- Instructor’s Confirmation

- The course instructor’s signature is required to verify the completion of the lab. Please ensure this is included with your report’s submission.

- Assignment Summary

- Provide a clear summary of the assignment, detailing the objectives, a description of the work completed, and your conclusions.

- Results Documentation

- Attach screenshots to present your results. Include your exported Node-RED .json flow file as part of your submission.

- The lab report will be assessed based on the lab marking rubric provided on Blackboard. Please review the rubric to ensure all criteria are met.

# Application Photos