MENG 4010 - INDUSTRIAL NETWORKING

INTERFACING IFM AL1320 IO-LINK MASTER WITH CONTROLLOGIX PLC1756-L81E

Student Name & Number:

Lab Date:

# Objective

This lab introduces the process of integrating an IFM AL1320 IO-Link Master with a ControlLogix 1756-L81E PLC over EtherNet/IP using Studio 5000. Students will configure network settings, import device data via an EDS file, and use Add-On Instructions (AOIs) to read sensor values into the PLC program.

Upon successful completion, students will be able to:

- Assign a static IP address to the IFM AL1320 IO-Link Master.

- Configure the EtherNet/IP interface in Studio 5000 using the EDS file.

- Import and apply AOIs for reading IO-Link sensor data.

- Create PLC tags to store and monitor sensor outputs.

- Validate successful communication and sensor functionality.

# Background

IO-Link is an open standard for point-to-point communication between sensors/actuators and controllers. Unlike traditional digital I/O, IO-Link supports diagnostics, identification, and advanced parameter data over a 3-wire connection. The IFM AL1320 IO-Link Master acts as a gateway between IO-Link field devices and EtherNet/IP-enabled PLCs. This lab simulates a modern industrial automation scenario where IO-Link sensors are integrated into a centralized PLC for intelligent monitoring.

# Demonstration Requirements

Students will use the provided EDS file for the IFM AL1320 IO-Link Master along with the corresponding Add-On Instructions (AOIs) to develop a Studio 5000 PLC program. The program will be configured to read key parameters from each connected IO-Link sensor and store the values in appropriate PLC tags.

Main steps are listed as follows:

Verify AL1320 is accessible within the LAN (make sure to use the correct device)

- Import EDS File into Studio 5000

- Import Add-On Instructions

- Program Sensor Data Retrieval

- Validate Live Data

# Optional Extension Task (Bonus)

Create a Fault Detection Routine

Add logic to monitor Status bits of each sensor. If communication is lost or a sensor fails, turn ON corresponding alarm bits.

# Lab Report Requirements

The lab report must contain:

- Instructor’s Confirmation

- The course instructor’s signature is required to verify the completion of the lab. Please ensure this is included with your report’s submission.

- Assignment Summary

- Provide a clear summary of the assignment, detailing the objectives, a description of the work completed, and your conclusions.

- PLC Program Documentation

- Attach a PDF of the PLC program and the PLC pgrogram itself that you created, ensuring it includes your initials whenever applicable.

- The lab report will be assessed based on the lab marking rubric provided on Blackboard. Please review the rubric to ensure all criteria are met.

# Application Photos