ROBOT EMBEDDED PROGRAMING

Lab 5: Publisher and Subscriber

HUMBER POLYTECHNIC

FACULTY OF APPLIED SCIENCES AND TECHNOLOGY

Name:

Student #1

Student #2

Student #3

Student #4

## Objective

This lab manual guides you through creating and running a simple ROS 2 publisher and subscriber using Python. You will build a package, add dependencies, create entry points, and run the nodes to observe communication in action.

# Step 1: Prepare Your System and Source ROS 2 Jazzy

Open a terminal and run:

# Update and upgrade Ubuntu packages

sudo apt update && sudo apt upgrade -y

# Source ROS 2 Jazzy environment (adjust path if installed elsewhere)

source /opt/ros/jazzy/setup.bash

# Create the workspace

mkdir -p ~/ros2_ws/src

cd ~/ros2_ws/

colcon build

This ensures your system is up to date, ROS 2 is on your path, and the workspace structure exists.

# Step 2: Create a Package

- cd ~/ros2_ws/src

- ros2 pkg create --build-type ament_python first_package

# Step 3: Open VS

- Code .

# Step 4: Add Dependencies in package.xml

Open the file 'package.xml' inside your 'first_package’folder and add the following dependencies before “export”: (Make sure to SAVE)

<depend>rclpy</depend>

<depend>std_msgs</depend>

# Step 5: Create Publisher Node (demo.py)

Inside the 'first_package/first_package/' folder, create a file named demo.py with the following code: (Make sure to SAVE)

import rclpy

from rclpy.node import Node

from std_msgs.msg import String

class MinimalPublisher(Node):

def __init__(self):

super().__init__('minimal_publisher')

self.publisher_ = self.create_publisher(String, 'topic', 10)

self.timer = self.create_timer(0.5, self.timer_callback)

self.i = 0

def timer_callback(self):

msg = String()

msg.data = f'Hello, ROS 2: {self.i}'

self.publisher_.publish(msg)

self.get_logger().info(f'Publishing: "{msg.data}"')

self.i += 1

def main(args=None):

rclpy.init(args=args)

node = MinimalPublisher()

rclpy.spin(node)

node.destroy_node()

rclpy.shutdown()

if __name__ == '__main__':

main()

Similarly, Inside the 'first_package/first_package/' folder, create a file named listener.py with the following code: (Make sure to SAVE)

import rclpy

from rclpy.node import Node

from std_msgs.msg import String

class MinimalSubscriber(Node):

def __init__(self):

super().__init__('minimal_subscriber')

self.subscription = self.create_subscription(

String,

'topic',

self.listener_callback,

10)

def listener_callback(self, msg):

self.get_logger().info(f'I heard: "{msg.data}"')

def main(args=None):

rclpy.init(args=args)

node = MinimalSubscriber()

rclpy.spin(node)

node.destroy_node()

rclpy.shutdown()

if __name__ == '__main__':

main()

# Step 6: Add Entry Points in setup.py

Open 'setup.py' and add the following under 'entry_points': (Make sure to SAVE)

entry_points={

'console_scripts': [

'talker = first_package.demo:main',

'listener = first_package.listener:main',

],

},

# Step 7: Build the Workspace

- From the workspace root:

- cd ~/ros2_ws/

- # Source Jazzy again to ensure the build tools see ROS 2

- source /opt/ros/jazzy/setup.bash

- colcon build

- # Source the newly built workspace

- source install/setup.bash

# Step 8: Run the nodes

Open two terminals. In each terminal, always source ROS 2 Jazzy and your workspace before running:

source /opt/ros/jazzy/setup.bash

source ~/ros2_ws/install/setup.bash

Terminal 1 – Publisher

ros2 run first_package talker

Terminal 2 – Subscriber

ros2 run first_package listener

# Step 9: Deliverables

- Screenshots (or a short terminal recording) showing:

- ros2 run first_package talker output publishing messages.

- ros2 run first_package listener output receiving those messages.

Include the commands used to source the Jazzy environment and the workspace in each terminal.

- A short paragraph (3-5 sentences) describing what happens between the publisher and subscriber, and what you learned about ROS 2 communication.

- Inspect the running system. While both nodes are running, open a third terminal (sourced) and run:

ros2 node list

ros2 topic list

ros2 topic info /topic

ros2 topic hz /topic

rqt_graph

Include screenshots, then answer:

- What message type is used on /topic, and how many publishers and subscribers are attached?

- What publishing rate does ros2 topic hz report? Which line of demo.py sets this rate?

- Predict, then test. For each scenario, write your prediction first, then run it and record what happens:

  - (a) Start the listener first, wait 10 seconds, then start the talker. Does the listener receive message 0?

  - (b) Stop the talker (Ctrl+C) while the listener is still running. What happens to the listener?

  - (c) Change the topic name in listener.py from 'topic' to 'chatter', rebuild, and run both nodes. What happens, and why? Restore the original name afterward.